#define BLYNK_TEMPLATE_ID "TMPL3VJxQUV-f"

#define BLYNK_TEMPLATE_NAME "FIRE DETECTION"

#define BLYNK_AUTH_TOKEN "ge-I6bhy6EmeHe-dygd_RzxS6XMBXRwW"

#include <WiFi.h>

#include <BlynkSimpleEsp32.h>

#include <DHT.h>

// Wi-Fi credentials

char ssid[] = "Mahesh_3g";

char pass[] = "789251837";

// Sensor and actuator pins

const int MQ2_PIN       = 34;  // MQ2 analog pin

const int DHT_PIN       = 23;  // DHT11 data pin

const int BUZZER_PIN    = 4;   // Buzzer pin

const int GREEN_LED_PIN = 18;  // Wi-Fi status

const int RED_LED_PIN   = 19;  // Fire or sensor failure

DHT dht(DHT_PIN, DHT11);

// Thresholds

const int GAS_THRESHOLD = 400;

const float TEMP_THRESHOLD = 38;

const float HUMIDITY_LOW_THRESHOLD = 10;

BlynkTimer timer;

// Flags

bool fireAlertSent = false;

bool dhtFailSent = false;

bool mq2FailSent = false;

bool deviceEnabled = true; // Remote ON/OFF flag

// Remote ON/OFF control from Blynk V6

BLYNK_WRITE(V6) {

deviceEnabled = param.asInt();  // 1 = ON, 0 = OFF

}

void setup() {

Serial.begin(115200);

pinMode(BUZZER_PIN, OUTPUT);

pinMode(GREEN_LED_PIN, OUTPUT);

pinMode(RED_LED_PIN, OUTPUT);

dht.begin();

Blynk.begin(BLYNK_AUTH_TOKEN, ssid, pass);

timer.setInterval(2000L, sendSensorData); // every 2 seconds

}

void sendSensorData() {

if (!deviceEnabled) {

Serial.println("[INFO] Device disabled from Blynk.");

digitalWrite(BUZZER_PIN, LOW); // ensure buzzer is off

digitalWrite(RED_LED_PIN, LOW); // turn off red LED

return; // skip sensor reading

}

int gasValue = analogRead(MQ2_PIN);

float temperature = dht.readTemperature();

float humidity = dht.readHumidity();

Serial.print("MQ2 Gas: "); Serial.println(gasValue);

Serial.print("Temp: "); Serial.print(temperature);

Serial.print(" °C  Humidity: "); Serial.print(humidity); Serial.println(" %");

// Send values to Blynk

Blynk.virtualWrite(V0, temperature);

Blynk.virtualWrite(V1, humidity);

Blynk.virtualWrite(V2, gasValue);

bool dhtFail = isnan(temperature) || isnan(humidity);

bool mq2Fail = gasValue >= 4095;

bool fireDetected = gasValue > GAS_THRESHOLD && temperature > TEMP_THRESHOLD;

//  Fire condition

if (fireDetected) {

digitalWrite(BUZZER_PIN, HIGH);

digitalWrite(RED_LED_PIN, HIGH); // Solid red LED

if (!fireAlertSent) {

Blynk.logEvent("gas", " Flammable Gas Detected!");

Blynk.logEvent("temp", " High Temperature!");

fireAlertSent = true;

}

} else {

digitalWrite(BUZZER_PIN, LOW);

fireAlertSent = false;

}

//  Sensor health LED status

if (dhtFail || mq2Fail) {

digitalWrite(RED_LED_PIN, millis() / 300 % 2); // Blink red LED

} else if (!fireDetected) {

digitalWrite(RED_LED_PIN, LOW); // Turn off red LED

}

//  Sensor health display on Blynk

if (dhtFail) {

if (!dhtFailSent) {

Blynk.logEvent("sensor_dht_fail", "❌ DHT Sensor Failure");

dhtFailSent = true;

}

Blynk.virtualWrite(V3, "❌ FAIL");

} else {

dhtFailSent = false;

Blynk.virtualWrite(V3, "✅ OK");

}

if (mq2Fail) {

if (!mq2FailSent) {

Blynk.logEvent("sensor_mq2_fail", "❌ MQ2 Sensor Failure");

mq2FailSent = true;

}

Blynk.virtualWrite(V4, "❌ FAIL");

} else {

mq2FailSent = false;

Blynk.virtualWrite(V4, "✅ OK");

}

}

void loop() {

Blynk.run();

timer.run();

//  Green LED logic

if (!deviceEnabled) {

digitalWrite(GREEN_LED_PIN, LOW); // OFF when device is disabled

} else if (WiFi.status() == WL_CONNECTED) {

digitalWrite(GREEN_LED_PIN, HIGH); // Solid ON when connected

} else {

digitalWrite(GREEN_LED_PIN, millis() / 500 % 2); // Blinking when not connected

}

}